Hi Christine

See Operational report.

Carried out 2nd August 2022

All in good order.

As per phone conversation.

Cable way next to road could do with tension.

Photo attached of the spring connector on cable to slide spring back a little for more tension if required.

Odd small clumps grass getting bit large in rubber. Might have to weed kill after summer hols when no children around as might damage rubber pulling out or pull out carefully.

Couple sheiks in Robinia timber, not a hazard. Exceptional dryness, probably will close a bit when moister.

Stain left on porch, I will bill later for this.

All in the report.

Thanks Martin

LARS LAJ UK

Charlotte House

500 Charlotte Road

Sheffield

S2 4ER

Phone 0800 2922304

Mobile 07817 850050

Email larslaj@larslaj.co.uk

www.larslaj.co.uk

Company Number: 07496302

Christine Lewin

Cheyneys

Ludgershall

Aylesbury

Bucks

HP18 9PB

17th August 2022. Operational report carried out 2nd August Ludgershall play area.

Dear Christine

Please find an operational report on play area, carried out 2nd August.

Signage. Adequate signage for safety.

Train. Satisfactory.

BNS swing next to Train. Satisfactory. Make sure chaps check shackles regular for ware.

Cradle swing satisfactory.

Tunnels satisfactory.

Springers satisfactory.

Knights Castle toddler unit. Satisfactory

Wicksteed RA. Monitor, some minor Play on bearings as movement. Check weekly for more movement. No deterioration since last report.

Seesaw. Satisfactory.

Twister. Satisfactory.

Climbing net spinner. Satisfactory.

Spinner. Monitor, play and movement on bearings, recommend seek advice from original supplier, replacing bearings. Same deterioration as before.

New robinia trail satisfactory. Note Bridge post fairly large sheik ( split ) not a hazard but recommend filling. Note due to dry Summer making it worse. Should close up some when moist again.

BNS swing next exs spinner. Satisfactory. Monitor shackles at top bar weekly. Satisfactory.

Junior flat swings, satisfactory. Note check shackles regularly.

Exs red monkey bars. Satisfactory. Note Rynat stated that technically there should be another 300mm each side of it with grass matting but said it was a low risk. Comment from lars Monitor.

New fitness. Satisfactory.

Cable ways. Tension cable on near side to road. The stopper spring can be moved back slightly to gain more tension. See pic of spring tensioner. Other cable way near goal tension ok.

Surfacing satisfactory. Grass coming through on some areas of rubber, remove before established. As per info given by DCM who laid the rubber.

In General play area in satisfactory condition. No high risk problems were found, some extra staining of high use parts of the timber like foot rungs on bridges etc.

Note make sure inspected weekly due to the high volume of use.

Some photos are attached with ref to comments.

Kind Regards

Martin Wood consultant for Lars Laj UK & Ireland.

Ph. 07817850050